Appendix J

Environmental Requirements and Mitigation Plan

1. Main references and specifications to be considered

The Subcontractor shall and must comply with all of the requirements and specifications detailed and listed in the following:

- The Subcontract, the Main Contract and it’s attachment, including but not limited to Schedule 2 (Employer’s Requirements), Schedule 4 (Authorizations), Schedule 18 (HS Specification & Risk Assessment), Schedule 20 (Project Company ESG Guideline)

- The Environmental Authorizations (EAs) and Environmental Management Programme (EMPr) listed in Main Contract Schedule 4.

- South African Legal Framework (Mandatory)

- National Environmental Management Act (NEMA)

- Environmental Impact Assessment Regulations (2014)

- National Water Act

- National Environmental Management: Waste Act

- Air Quality Act

- Occupational Health and Safety Act (OHSA)

- Construction Regulations (2014)

(2) Contractual Requirements

- EPC Contract obligations

- ESIA and Environmental Authorizations

(3) International Standards

- IFC Performance Standards (2012)

- World Bank EHS Guidelines

- Equator Principles

- ISO 14001 & ISO 45001

2. Environmental and Social Management Basic Specification

In addition to the previously mentioned references, the Subcontractor shall prepare and deliver to the Contractor and shall be responsible for regularly maintaining and updating the HSESMP in accordance with the following:

- the Subcontractor's corporate risk-based HSE management system, based on Good Utility Practice, certified to the ISO 14001 and ISO45001 standards, and intended to assure the Project's compliance with local legislation, permits and contractual obligations; and

- the HSESMP is a Project‑ and Works‑specific risk‑based HSE and social management plan that combines and enacts the Project’s HSE management system and the Project’s obligations under the Environmental Authorizations, EMPr, ESG Guidelines, IFC Performance Standards and the Equator Principles. It shall be established and maintained from Site mobilization until the Taking‑Over Certificate is issued, and thereafter continue during the Defects Notification Period as necessary, to plan, control, manage, monitor, report on and enforce HSE and social issues.

The Project‑specific HSESMP shall be aligned with and comply with Good International Industry Practice (GIIP) and, for the avoidance of doubt, the Contractor’s benchmarks for GIIP include, but are not limited to:

- IFC Performance Standards.

- World Bank Group EHS Guidelines.

- Equator Principles (July 2020).

- South African Construction Regulations, 2014 (GNR. 84 of 7 February 2014).

- Relevant SANS codes and industry best practice for solar PV construction.

The Project’s HSESMP shall cover and address HSE and environmental management and performance issues as applicable to the technology and processes for project execution. The minimum required contents are specified in the Employer’s Requirements and Schedule 20 (Project Company ESG Guideline).

If any specific need is identified during project execution, additional management plans (e.g., Waste Management Plan, Water Management Plan, Biodiversity Management Plan, Community Grievance Mechanism) shall be developed as required.

3. Basic requirements for Subcontractor’s Environmental Management Personnel

The Subcontractor shall, prior to the commencement of the Works at the Site mobilize sufficient and adequate resources and implement the Project’s risk based HSE management system and plan.

The Subcontractor's on-site staff shall include environmental management personnel, who shall be appropriately qualified as detailed in the table below: “Competence requirements for EPC Environmental Management Personnel”. Deviations from the below minimum requirements are by exception and with the Contractor’s prior written approval. The number of personnel assigned to these tasks shall be appropriate to the extent, complexity and regulatory context in which the project is being developed.

All Subcontractor’s HSE and Environmental Management personnel shall be interviewed, assessed and approved by the Contractor and the Owner before being hired for the Project.

The Subcontractor shall, and shall procure that its agents, representatives, Sub-subcontractors of any tier in each case involved in the Project shall, comply with, and carry out the Project in accordance with, all applicable Environmental, Health and Safety Laws. Please see the following table for reference:

| Position | Qualification | Professional Accreditation | HSE Experience | English Fluency | Contractor and Owner Approval |
| --- | --- | --- | --- | --- | --- |
| Environmental Engineer | Bachelor’s degree in Environmental management | Full Membership of professional organization/ institute SACPCMP registration preferred; IEMA or equivalent | >10 years in power/ renewable industry and construction sector(s) as an environmental professional | CEFR – B2, Independent User, Upper Intermediate | Interview before arrival at site and 3 month probation period |

4. Reporting

The Subcontractor shall prepare an HSE management and performance report regarding the on-going works on the Site on a monthly and quarterly basis. Each report shall be delivered to the Contractor by no later than the fifth (5th) Day of the month following the end of the relevant quarter and contain the following data as per the Contractor’s HSE Monitoring and Reporting procedure.

| Environmental | Monthly | YTD | Remarks (if Any) |
| --- | --- | --- | --- |
| Environmental samples – number analyzed |  |  |  |
| Environmental samples – % compliant with license |  |  | Samples must be taken according to the environmental license requirements, aligned with local regulation. |
| Hazardous substance quantity of spillage (liter/kg) |  |  | Only in case of a spill according to the local regulation. |
| Waste disposed - fly ash (kg) |  |  | N/A |
| Waste disposed - hazardous waste (kg) |  |  |  |
| Waste disposed - general/non-hazardous waste (kg) |  |  |  |
| HTF contaminated water (m3) – only CSP plants |  |  | N/A |
| Waste disposed –HTF contain soil & waste (kg) – only CSP |  |  | N/A |
| Waste disposed - radioactive waste (kg) |  |  | N/A |
| Waste disposed - medical waste (kg) |  |  |  |
| Waste recycled plastic (kg) |  |  |  |
| Waste recycled glass (kg) |  |  |  |
| Waste recycled oil & grease (kg) |  |  |  |
| Waste recycled wood (kg) |  |  |  |
| Waste recycled metal (kg) |  |  |  |
| Waste recycled paper & packaging (kg) |  |  |  |
| Waste recycled other (kg) |  |  |  |
| Waste recycled HTF (m3) - – only CSP plants |  |  | N/A |

In addition to these statistics, the Subcontractor shall report the following KPI related to sustainability (as applicable to the project’s technology):

Air quality and emissions (PM₁₀, PM₂.₅, SO₂, NO₂, CO)

Ambient noise

Waste water discharge (pH, temperature, turbidity, dissolved oxygen, residual chlorine)

Water consumption (m³) from municipal, underground and surface sources

Energy consumption (electricity, diesel, petrol, etc.)

Greenhouse gas emissions (Scope 1 and 2) – CO₂

Community grievances received and resolved

Local procurement spend (ZAR) and job creation numbers.

LTA environmental report during construction

The Employer shall procure that the LTA, no later than 45 days after each quarter (commencing from Financial Close until Taking‑Over), delivers to the Lenders a quarterly construction report verifying the Employer’s compliance with its environmental, health and safety obligations under the Finance Documents and the Contract, and detailing any material non‑compliance with IFC Performance Standards, the ESMS or the EMPr. The Subcontractor shall provide all necessary access and information to the LTA for this purpose.

5. Additional requirements

Additional standards to be considered when developing project HSE Management Strategies, Objectives and Management/Mitigation Programs are:

“Environmental Standards” means each of those standards applicable in relation to the Company and/or Project under:

- the World Bank Environmental Standards;

- the IFC Performance Standards;

- Environmental, Health and Safety Guidelines (effective April 2007);

- the Equator Principles;

- Environmental and Social Law and/or local regulation from any interested third party;

- E&S Action Plan; and

- any international convention relating to the Environment to which South Africa is the signatory and which is applicable to the Company, the Project or the subject matter of, or any party (in relation to the Project) to, any of the Transaction Documents.

“Equator Principles” means the Principles promulgated on 4 June 2003 (as amended and modified, effective 6 July 2006 and on 4 June 2017) as available and available, as at the date of this Contract on https://equator-principles.com  and as may be amended and supplemented from time to time

“IFC Performance Standards” means the IFC's Performance Standards on Environmental and Social Sustainability, dated 1 January 2012.

World Bank Environmental Standards means the environmental standards, requirements and guidelines contained in the Policy on Environmental and Social Sustainability published by the International Finance Corporation and effective 1 January 2012.

6. Audit requirements

1. Obligation to Cooperate Unconditionally: The Subcontractor acknowledges that the Works may be subject to audits by the Contractor, the Owner, or relevant government agencies. The Subcontractor shall provide full, unconditional, and continuous cooperation, assistance, and access to all records, accounts, documents, and personnel as may be requested to facilitate such audits.

- Binding Effect of Third-Party Audits: The Subcontractor expressly agrees to be bound by the results of any audit conducted by the Owner or relevant government authorities. The Contractor shall have the right to adjust the Subcontractor’s payments or impose deductions based on such audit findings. The Subcontractor waives any right to challenge the validity of the Employer's or government's audit conclusions.

- Costs and Penalties: Any costs, fines, or disallowed amounts arising from the Subcontractor’s failure to comply with audit requirements or from inaccuracies in the Subcontractor’s records shall be solely borne by the Subcontractor. Failure to provide prompt cooperation shall constitute a material breach of this Subcontract.